Bridge & Crossing Inventory: Road-Waterway Intersection Points Task brief for GIS analyst

Reliable transport access depends on knowing where road infrastructure interacts with drainage and river systems. You are required to perform an analysis that will produce a point layer of all locations where the road network crosses a waterway within the project area, creating a candidate inventory of possible bridges, culverts, and drainage crossing assets. The output will support infrastructure audits by helping planners identify crossing points that may require field verification, structural assessment, maintenance prioritization, or flood-risk review. This provides a practical spatial baseline for improving road resilience, asset management, and investment planning.

You will be working with three layers: road_networks_Bo.gpkg and water_lines_Bo.gpkg  as the primary analysis inputs, and SLE_adm2_Bo.gpkg, included solely to provide administrative boundary context for the map. The layers must be in EPSG:32628 (WGS 84 / UTM Zone 28N) before any processing begins. You must verify and reproject, if necessary (use the reprojected layers for all analysis), as all area and length calculations depend on metric units.

The core operation is a line intersection. Using the Vector Overlay and running the Line Intersections tool with road_networks_Bo.gpkg set as the input layer and water_lines_Bo.gpkg  as the intersect layer. The output is saved as bridge_crossing_points.gpkg, a point layer where each point represents a location at which a road segment and a waterway segment cross, preserving all available attributes from both source layers in the output, road attributes with their default field names, and waterway attributes prefixed with water_ to avoid field name collisions. Do not apply any pre-filtering; the intent is to capture every crossing across the full dataset, so the inventory is exhaustive.

Deliver a project folder, Water-Road_Crossing, containing the input layers (water_lines_Bo.gpkg , road_networks_Bo.gpkg, and SLE_adm2_Bo.gpkg), the derived output layer (bridge_crossing_points.gpkg), and Bridge_Crossing_Inventory.qgz saved with a relative path, and reopen without broken layer links, with all three layers visible and correctly symbolized, crossings as distinct point markers, roads and waterways as contrasting line colours.