Road-Waterway Crossing Risk Register: Bo District Infrastructure Audit Task brief for GIS analyst

Reliable transport access depends on knowing where road infrastructure interacts with drainage and river systems. You are required to perform an analysis that produces a classified point inventory of all locations where the road network crosses a waterway within the Bo District project area, identifying candidate bridges, culverts, and drainage crossing assets and assigning each a risk tier based on the road class it serves and the waterway type it crosses.

You will be working with three layers: road_networks_Bo.gpkg and water_lines_Bo.gpkg  as the primary analysis inputs, and SLE_adm2_Bo.gpkg, included solely to provide administrative boundary context for the map. The layers must be in EPSG:32628 (WGS 84 / UTM Zone 28N) before any processing begins. You must verify and reproject, if necessary (use the reprojected layers for all analysis), as all area and length calculations depend on metric units.

The core operation is a line intersection. Using the Vector Overlay and running the Line Intersections tool with road_networks_Bo.gpkg set as the input layer and water_lines_Bo.gpkg  as the intersect layer. The output is saved as bridge_crossing_points.gpkg, a point layer where each point represents a location at which a road segment and a waterway segment cross, preserving all available attributes from both source layers in the output, road attributes with their default field names, and waterway attributes prefixed with water_ to avoid field name collisions. Do not apply any pre-filtering; the intent is to capture every crossing across the full dataset, so the inventory is exhaustive.

Enrich the bridge_crossing_points.gpkg with five calculated fields directly in the attribute table and save as bridge_crossing_classified.gpkg:

With the RTT_DESCRI field, which carries the road classification: map Primary Route to road_tier = 1 and road_tier_label = Tier 1 - Primary Route, and Secondary Route to road_tier = 2 and road_tier_label = Tier 2 - Secondary Route.

Clean the water_waterway field into a waterway_type field using plain labels: river becomes Major - River, stream becomes Minor - Stream, and drain becomes Minor - Drain.

Assign a crossing_id using a sequential code formatted as BCI-001 through BCI-030.

Derive a risk_tier field by combining road tier and waterway type: a Tier 1 road over a river is Critical; a Tier 1 road over a stream or drain, or a Tier 2 road over a river, is High; all remaining combinations are Moderate. These five fields together transform the raw geometry into a decision-ready inventory.

Deliver a project folder, Water-Road_Crossing, containing the input layers (water_lines_Bo.gpkg , road_networks_Bo.gpkg, and SLE_adm2_Bo.gpkg), the derived output layers (bridge_crossing_points.gpkg, and bridge_crossing_classified.gpkg styled using the risk_tier field: red,  orange, and yellow marker for Critical,  High, and Moderate, respectively), and Classified_Bridge_Crossing_Inventory.qgz saved with a relative path, and reopen without broken layer links, with all three layers visible and correctly symbolized, crossings as distinct point markers, roads and waterways as contrasting line colours.