Road-Waterway Crossing Risk Register: Bo District Infrastructure Audit Task brief for GIS analyst

Reliable transport access depends on knowing where road infrastructure interacts with drainage and river systems. You are required to perform an analysis that produces a classified point inventory of all locations where the road network crosses a waterway within the Bo District project area, identifying candidate bridges, culverts, and drainage crossing assets. The objective is to rank every crossing by the structural and hydraulic demand that its waterway places on whatever bridge or culvert infrastructure sits there, so that the audit team can prioritize physical inspections by hydraulic exposure rather than treating all crossings equally.

You will be working with three layers: road_networks_Bo.gpkg and water_lines_Bo.gpkg  as the primary analysis inputs, and SLE_adm2_Bo.gpkg, included solely to provide administrative boundary context for the map. The layers must be in EPSG:32628 (WGS 84 / UTM Zone 28N) before any processing begins. You must verify and reproject, if necessary (use the reprojected layers for all analysis), as all area and length calculations depend on metric units.

The core operation is a line intersection. Using the Vector Overlay and running the Line Intersections tool with road_networks_Bo.gpkg set as the input layer and water_lines_Bo.gpkg  as the intersect layer. The output is saved as bridge_crossing_points.gpkg, a point layer where each point represents a location at which a road segment and a waterway segment cross, preserving all available attributes from both source layers in the output, road attributes with their default field names, and waterway attributes prefixed with water_ to avoid field name collisions. Do not apply any pre-filtering; the intent is to capture every crossing across the full dataset, so the inventory is exhaustive.

The bridge_crossing_points.gpkg layer already carries water_waterway and water_name attributes inherited from the water_lines_Bo.gpkg source layer during the line intersection step. Score each crossing point based on its water_waterway value: rivers carry the highest hydraulic demand and score 4, streams are moderate and score 2, and drains are the lowest and score 1. Store the score as magnitude_score and a respective label: Major - River, Minor - Stream, and Minor – Drain, as hydraulic_class.

Each crossing point already holds a water_fid attribute that references the waterway segment it sits on. Use that key to look up the length of the corresponding waterway segment from water_lines_Bo.gpkg using Join Attributes by Field Value, and store it as waterway_length_m. Longer waterways carry larger drainage areas and higher peak flows, so segment length serves as a continuous size indicator to complement the type-based score.

With both fields populated, populate the hydraulic_rank field by sorting all crossings descending on magnitude_score first, then waterway_length_m as a tiebreaker, assigning rank 1 to the highest-exposure crossing. Save the output as waterway_magnitude_crossings.gpkg, styled using graduated symbology on hydraulic_rank with 3 equal interval classes and a dark red to light yellow colour ramp -- rank 1 rendered as the darkest and largest marker -- so that High, Moderate, and Low hydraulic exposure are immediately readable on the map canvas.

Deliver a project folder, Water-Road_Crossing, containing the input layers (water_lines_Bo.gpkg, road_networks_Bo.gpkg, and SLE_adm2_Bo.gpkg), the derived output layers (bridge_crossing_points.gpkg, and waterway_magnitude_crossings.gpkg), and Magnitude_Bridge_Crossing_Inventory.qgz saved with a relative path, and reopen without broken layer links, mismatch, or missing CRS.