Flood Exposure Ranking of Road-Waterway Crossings: Bo District Infrastructure Audit Task brief for GIS analyst

Reliable transport access depends on knowing where road infrastructure interacts with drainage and river systems. You are required to produce a fully attributed point layer of all road-waterway crossing locations within the Bo District, Sierra Leone, each scored with a composite flood inundation risk tier, to serve as a candidate bridge and culvert inventory for infrastructure audit and risk prioritization. The objective is to locate every point where a road crosses a waterway in the district and rank each crossing by its flood risk, so infrastructure teams know which bridges and culverts to prioritise for inspection and intervention.

You will be working with three layers: road_networks_Bo.gpkg and water_lines_Bo.gpkg as the primary analysis inputs, and SLE_adm2_Bo.gpkg, which is included solely to provide administrative boundary context for the map. The layers must be in EPSG:32628 (WGS 84 / UTM Zone 28N) before any processing begins. You must verify and reproject, if necessary (use the reprojected layers for all analysis), as all area and length calculations depend on metric units.

The core operation is a line intersection. Using the Vector Overlay and running the Line Intersections tool with road_networks_Bo.gpkg set as the input layer and water_lines_Bo.gpkg  as the intersect layer. The output is saved as bridge_crossing_points.gpkg, a point layer where each point represents a location at which a road segment and a waterway segment cross, preserving all available attributes from both source layers in the output, road attributes with their default field names, and waterway attributes prefixed with water_ to avoid field name collisions. Do not apply any pre-filtering; the intent is to capture every crossing across the full dataset, so the inventory is exhaustive.

To produce the flood vulnerability layer, each crossing point in bridge_crossing_points.gpkg must carry five new fields appended to its original attributes: waterway_score, road_score, combined_score, flood_tier_num, and flood_tier_label, and the enriched layer is saved as flood_vulnerability_crossings.gpkg. The scoring approach uses two attributes already carried on each crossing point: the type of waterway being crossed (water_waterway) and the road classification of the road doing the crossing (road_tier).

Assign a waterway magnitude score to the waterway_score field based on the water_waterway value: river = 3, stream = 2, drain = 1. Assign a road importance score to the road_score field based on the road_tier value: road_tier 1 (primary route) = 2, road_tier 2 (secondary route) = 1. Sum these two values into the combined_score field -- the minimum possible combined score is 2, and the maximum is 5.

Map the combined_score to flood_tier_num and flood_tier_label as follows: a combined score of 5, which can only result from a primary route crossing a river, maps to flood_tier_num 1 with the label CRITICAL. A combined score of 4, which results from either a secondary route crossing a river or a primary route crossing a stream, maps to flood_tier_num 2 with the label HIGH. A combined score of 3, which results from either a secondary route crossing a stream or a primary route crossing a drain, maps to flood_tier_num 3 with the label MODERATE. A combined score of 2, which can only result from a secondary route crossing a drain, maps to flood_tier_num 4 with the label LOW.

Deliver a project folder, Water-Road_Crossing, containing the input layers (water_lines_Bo.gpkg, road_networks_Bo.gpkg, and SLE_adm2_Bo.gpkg), the derived output layers (bridge_crossing_points.gpkg, and flood_vulnerability_crossings.gpkg styled with categorized symbology on the flood_tier_label field with “viridis” color ramp in ascending order), and Bridge_Crossing_Inventory.qgz saved with a relative path, and reopen without broken layer links, mismatch, or missing CRS.